| # AIN SHAMS UNIVERSITY FACULTY OF ENGINEERING IRRIGATION & HYDRAULICS DEPT. | # 4th. Year Civil Groundwater Hydrology 2015-2016 |
| --- | --- |

Assignment (3)

1	put (x) or (√) and correct the wrong sentence:

- aquifer yield water more than aquitarde

- Aquifer gives water more than aquiclude

- Hydraulic conductivity in unconfined aquifer is calculated by multiplying transmissivity by the total aquifer thickness

- Specific storage is less than specific yield

- Intrinsic permeability has dimensions [L/T]

2	A well is pumped from confined aquifer at a rate of 0.08 m3/s for a long time. In two observation wells located 50 m and 10 m away from the well, the difference in elevation has been observed as 1.5 m. what is the transmissivity of the aquifer?

3	Figure (3-1) shows a steady state of flow under a uniform infiltration rate of value e. if the water level above the impervious base in rivers A and B is 20m and the elevation of the water surface in the observation well is 21 m.

Calculate:

- the ratio e/K, (K is hydraulic conductivity)

- the maximum water depth between the two rivers.

Figure (3-1)

4	Calculate the hydraulic conductivity of an artesian (confined) aquifer. The aquifer thickness is 27 m. the well discharge is 0.05 m3/sec. the drawdown in an observation well 15 m away is 3 m and in another well 150 m away is 0.3 m.

5	An aquifer of 20 m average thickness is overlain by an impermeable layer of 30 m thickness. A test well of 0.5 m diameter and two observation wells at a distance of 10 m and 60 m from the test well are drilled through the aquifer. After pumping at a rate of 0.1 m3/s for a long time, the following drawdowns are stabilized in these wells: first observation well, 4 m; second observation well, 3 m. Determine the hydraulic conductivity and drawdown in the test well.